Comparison Report

1. Company Comparison

| Company 1 | Company 2 |
| --- | --- |
| Farmism Foods LLP, Guwahati Assam | Maharashtra Foods (Amandan Foods LLP) |
| Recipe No. 3890 | Recipe No. 3890 |

2. Moisture Data Comparison

| Stage | Farmism Foods LLP | Maharashtra Foods LLP |
| --- | --- | --- |
| After Mixing | 11.38 | 11.09 |
| After Conditioning | 12.38 | 13.21 |
| After Pellet | 11.75 | 12.95 |
| After Cooler | 10.56 | 10.81 |
| After Bagging | 10.52 | 10.36 |

3. Machine Parameters

| Parameter | Farmism Foods LLP | Maharashtra Foods LLP |
| --- | --- | --- |
| Boiler Pressure | 6.5 bar | 8.5 bar |
| After PRV | 2 bar | 2 bar |
| Conditioner Temp | 88°C, 82°C | 84°C |
| Pellet Feeder RPM | 154 | 31 |
| Pellet Mill Load | 190 amp | 336 amp |

4. Site Inspection Notes of Amardan foods LLP

1. Pipe fit in Mixer for water addition (1 inch pipe) is not used when recipe starts for finished goods(De hues).It's used only for other's client recipes.

2. De hues bags , other clients bag and rejected material also dorb bags are near the plant.

3.Conditioner starts direct with motor by flange couplings.( Size is also bigger as compared to our company).

4. Pellet size of finish good 4.5mm and length of finish good 15-20mm.